Cornell University, School of Electrical and Computer ENgineering

One-Dimensional Cellular Automaton

ECE5760

Jeremy Blum, Sima Mitra, Jason Wright

Thursday Lab Section

A binary, nearest-neighbor, one dimensional cellular automaton was calculated by a state machine running on an FPGA to be displayed on a connected VGA monitor at a resolution of 640x480 pixels.

# Introduction

The objective of this lab was to develop a 1-D cellular automaton that could be displayed on a VGA monitor at a resolution of 640x480.  An Altera DE-2 board with a Cyclone II FPGA was used to build a hardware state machine paired with a VGA driver and appropriate memory management to permit on-the-fly calculation of the cellular automaton states.  The cellular automaton rule is entered using the slider switches; the seed row can be set to a single pixel in the middle or a random row of pixel values.  An “advance” button shows the next 480 lines of evolution, and a reset button starts a new evolution with the settings specified by the switches.  The 7 segment displays are used to show the current rule setting.

# Design and Testing Methods

Below, we describe each of the key components to the larger system in detail, and how we tested each individual component before piecing it together into the complete cellular automaton system.

## Inputs and Outputs

We used a total of nine switches.  Switches zero through seven are used to input an 8-bit binary value to serve as the rule for the cellular automaton.  Switch 17 is used to toggle between using a single pixel starting point or a random distribution of pixels in the top row.  To confirm that the switches were being read properly by the program, the seven segment LED displays and the RED LEDs were configured to show the state of the switches and the state of the program.  The seven segment displays show the binary value that has been set by the rule switches, and some of the red LEDs were configured to show which state the program is current executing.  Figure 1 below shows a rule selected on the switches and confirmed on the seven segment displays:

Figure 1: Switch States shown on Seven Segment Displays

Key 0 was used to reset the state machine and Key 3 was used to advance the VGA display to the next line.  Since the buttons on the DE-2 board sometimes do not work well, they were configured to illuminate the green LEDs above them when pressed.  This provided useful confirmation that the button presses were successful.

## High-Level Structure

Figure 2 below shows the basic high-level system architecture:

Figure 2: High-Level System Organization Diagram

The state machine is written in Verilog and is built into the FPGA hardware.  Our design calculates the entire next row of pixels at once based on the previous row and the cellular automaton rule using the M4K memory.  This parallel computation is implemented by using 640 single automaton hardware units to compute the value of each pixel using data from the previous row.  The VGA image buffer is stored in M4K blocks, which are just large enough to hold a 640x480 image in addition to the cellular states.  The state machine is wired to the pushbutton and switch inputs for controlling the rule selection, random/single pixel state, reset, and “advance” functions.

## State Machine

Figure 3 shows the state machine that was implemented in hardware using Verilog.  The state machine is responsible for responding to keypresses, reading the automaton rule, and generating the contents to be sent to the VGA display adapter.  The three print cases are necessary to insert a timing delay for the VGA writes, and the “chill” states are where the program sits when it is waiting for user input to either reset the program or advance it to the next screen.  Any time key 0 is pressed, the state machine will be reset to the initial state.

Figure 3: State Machine Diagram

## Automaton Hardware

Figure 4Figure 4 shows the process for computing an entire row of automaton values in parallel.

Figure 4: Automaton Row-by-Row Calculation

Each row of pixel values is stored in a large 642-bit array.  The additional two bits are used as zero padding for the pixels that lie at the edges of the screen. For initial testing, the switches 17-15 were used to enter three values for the cellular automaton rule to operate on and the output of a single computation was displayed on the VGA in one pixel. By comparing the correct outcome of the cellular automaton with the output displayed allowed us to verify the computation.

## VGA

The VGA controller onboard the DE2 board is connected to a pre-allocated 640x480 block within M4K, accessed via addr_reg and data_reg pointers. Computed data is pushed into the VGA registers from registers on the Cyclone II chip, and the VGA controller accesses the M4K memory at 60 Hz to refresh the screen.

## Random Seed generation

20 32-bit linear feedback shift registers were used to generate a total of 640 pixels for the randomized first row when the appropriate selector switch was activated.  A conglomeration of 32-bit registers were used instead of one large 640 bit register because there is no known pseudo-random-generating 640-bit-wide linear feedback shift register.  Figure 5 below shows how one of these 32-bit linear feedback shift registers works:

Figure 5: 32-Bit LFSR

In Figure 6 below, you can see how the values from 20 LFSR’s were chained together to generate 640 random bits of data for the first seed row.

Figure 6: Chaining 20 LFSRs together

# Documentation

## Example Images

Below we show an example of rule 26 with a single seed pixel, which makes a repeating triangle tessellation.

The next figure shows rule 30 with a single seed pixel.

The next figure shows the same rule applied to a randomized seed row.

# Comparison of 1-D Cellular Automaton with Natural Phenomenon

## Seashells

The pigmentation of certain seashells is governed by cellular automaton-like rules: The shell of the Conus Textile is not unlike rule 30:

Figure 7: Conus Textile Seashell (Source: Wikipedia)

Figure 8: Rule 30 (Source: Wolfram Alpha)

## Caterpillars

For caterpillars, rule 105 is similar to the pattern on the head of the caterpillar, but the pattern on the body does not appear to be governed by a 1-dimentional cellular automaton rule.

Figure 9: A caterpillar with a cellular automaton like pattern. (Source: http://www.kathryncramer.com/kathryn_cramer/2005/09/cellular_automa.html)

Figure 10: Rule 105 (Source: Wolfram Alpha)

## Other 1-D Growth Systems

Rule 184 is used to model surface deposition in certain crystalline structures (D. de Cogan and L.R. Martin, "Simple models for crystallisation processes," Intl. Conference on Solid State Crystals, 2000.). Cellular automatons are diffusion-limited aggregation are both examples of one-dimensional growth systems.

# Program Listing

// --------------------------------------------------------------------

//

// Major Functions: Cellular Automaton

//      state is in m4k blocks

// --------------------------------------------------------------------

module DE2_TOP (

// Clock Input

input         CLOCK_27,    // 27 MHz

input         CLOCK_50,    // 50 MHz

input         EXT_CLOCK,   // External Clock

// Push Button

input  [3:0]  KEY,         // Pushbutton[3:0]

// DPDT Switch

input  [17:0] SW,          // Toggle Switch[17:0]

// 7-SEG Display

output [6:0]  HEX0,        // Seven Segment Digit 0

output [6:0]  HEX1,        // Seven Segment Digit 1

output [6:0]  HEX2,        // Seven Segment Digit 2

output [6:0]  HEX3,        // Seven Segment Digit 3

output [6:0]  HEX4,        // Seven Segment Digit 4

output [6:0]  HEX5,        // Seven Segment Digit 5

output [6:0]  HEX6,        // Seven Segment Digit 6

output [6:0]  HEX7,        // Seven Segment Digit 7

// LED

output [8:0]  LEDG,        // LED Green[8:0]

output [17:0] LEDR,        // LED Red[17:0]

// UART

output        UART_TXD,    // UART Transmitter

input         UART_RXD,    // UART Receiver

// IRDA

output        IRDA_TXD,    // IRDA Transmitter

input         IRDA_RXD,    // IRDA Receiver

// SDRAM Interface

inout  [15:0] DRAM_DQ,     // SDRAM Data bus 16 Bits

output [11:0] DRAM_ADDR,   // SDRAM Address bus 12 Bits

output        DRAM_LDQM,   // SDRAM Low-byte Data Mask

output        DRAM_UDQM,   // SDRAM High-byte Data Mask

output        DRAM_WE_N,   // SDRAM Write Enable

output        DRAM_CAS_N,  // SDRAM Column Address Strobe

output        DRAM_RAS_N,  // SDRAM Row Address Strobe

output        DRAM_CS_N,   // SDRAM Chip Select

output        DRAM_BA_0,   // SDRAM Bank Address 0

output        DRAM_BA_1,   // SDRAM Bank Address 0

output        DRAM_CLK,    // SDRAM Clock

output        DRAM_CKE,    // SDRAM Clock Enable

// Flash Interface

inout  [7:0]  FL_DQ,       // FLASH Data bus 8 Bits

output [21:0] FL_ADDR,     // FLASH Address bus 22 Bits

output        FL_WE_N,     // FLASH Write Enable

output        FL_RST_N,    // FLASH Reset

output        FL_OE_N,     // FLASH Output Enable

output        FL_CE_N,     // FLASH Chip Enable

// SRAM Interface

inout  [15:0] SRAM_DQ,     // SRAM Data bus 16 Bits

output [17:0] SRAM_ADDR,   // SRAM Address bus 18 Bits

output        SRAM_UB_N,   // SRAM High-byte Data Mask

output        SRAM_LB_N,   // SRAM Low-byte Data Mask

output        SRAM_WE_N,   // SRAM Write Enable

output        SRAM_CE_N,   // SRAM Chip Enable

output        SRAM_OE_N,   // SRAM Output Enable

// ISP1362 Interface

inout  [15:0] OTG_DATA,    // ISP1362 Data bus 16 Bits

output [1:0]  OTG_ADDR,    // ISP1362 Address 2 Bits

output        OTG_CS_N,    // ISP1362 Chip Select

output        OTG_RD_N,    // ISP1362 Write

output        OTG_WR_N,    // ISP1362 Read

output        OTG_RST_N,   // ISP1362 Reset

output        OTG_FSPEED,  // USB Full Speed, 0 = Enable, Z = Disable

output        OTG_LSPEED,  // USB Low Speed,  0 = Enable, Z = Disable

input         OTG_INT0,    // ISP1362 Interrupt 0

input         OTG_INT1,    // ISP1362 Interrupt 1

input         OTG_DREQ0,   // ISP1362 DMA Request 0

input         OTG_DREQ1,   // ISP1362 DMA Request 1

output        OTG_DACK0_N, // ISP1362 DMA Acknowledge 0

output        OTG_DACK1_N, // ISP1362 DMA Acknowledge 1

// LCD Module 16X2

inout  [7:0]  LCD_DATA,    // LCD Data bus 8 bits

output        LCD_ON,      // LCD Power ON/OFF

output        LCD_BLON,    // LCD Back Light ON/OFF

output        LCD_RW,      // LCD Read/Write Select, 0 = Write, 1 = Read

output        LCD_EN,      // LCD Enable

output        LCD_RS,      // LCD Command/Data Select, 0 = Command, 1 = Data

// SD Card Interface

inout         SD_DAT,      // SD Card Data

inout         SD_DAT3,     // SD Card Data 3

inout         SD_CMD,      // SD Card Command Signal

output        SD_CLK,      // SD Card Clock

// I2C

inout         I2C_SDAT,    // I2C Data

output        I2C_SCLK,    // I2C Clock

// PS2

input         PS2_DAT,     // PS2 Data

input         PS2_CLK,     // PS2 Clock

// USB JTAG link

input         TDI,         // CPLD -> FPGA (data in)

input         TCK,         // CPLD -> FPGA (clk)

input         TCS,         // CPLD -> FPGA (CS)

output        TDO,         // FPGA -> CPLD (data out)

// VGA

output        VGA_CLK,     // VGA Clock

output        VGA_HS,      // VGA H_SYNC

output        VGA_VS,      // VGA V_SYNC

output        VGA_BLANK,   // VGA BLANK

output        VGA_SYNC,    // VGA SYNC

output [9:0]  VGA_R,       // VGA Red[9:0]

output [9:0]  VGA_G,       // VGA Green[9:0]

output [9:0]  VGA_B,       // VGA Blue[9:0]

// Ethernet Interface

inout  [15:0] ENET_DATA,   // DM9000A DATA bus 16Bits

output        ENET_CMD,    // DM9000A Command/Data Select, 0 = Command, 1 = Data

output        ENET_CS_N,   // DM9000A Chip Select

output        ENET_WR_N,   // DM9000A Write

output        ENET_RD_N,   // DM9000A Read

output        ENET_RST_N,  // DM9000A Reset

input         ENET_INT,    // DM9000A Interrupt

output        ENET_CLK,    // DM9000A Clock 25 MHz

// Audio CODEC

inout         AUD_ADCLRCK, // Audio CODEC ADC LR Clock

input         AUD_ADCDAT,  // Audio CODEC ADC Data

inout         AUD_DACLRCK, // Audio CODEC DAC LR Clock

output        AUD_DACDAT,  // Audio CODEC DAC Data

inout         AUD_BCLK,    // Audio CODEC Bit-Stream Clock

output        AUD_XCK,     // Audio CODEC Chip Clock

// TV Decoder

input  [7:0]  TD_DATA,     // TV Decoder Data bus 8 bits

input         TD_HS,       // TV Decoder H_SYNC

input         TD_VS,       // TV Decoder V_SYNC

output        TD_RESET,    // TV Decoder Reset

// GPIO

inout  [35:0] GPIO_0,      // GPIO Connection 0

inout  [35:0] GPIO_1       // GPIO Connection 1

);

//Turn off all displays.

//assign HEX0 = 7'h7F;

//assign HEX1 = 7'h7F;

//assign HEX2 = 7'h7F;

//assign HEX3 = 7'h7F;

//assign HEX4 = 7'h7F;

//assign HEX5 = 7'h7F;

//assign HEX6 = 7'h7F;

//assign HEX7 = 7'h7F;

//assign LEDR = 18'h0;

//assign LEDG = 9'h0;

//Set all GPIO to tri-state.

assign GPIO_0 = 36'hzzzzzzzzz;

assign GPIO_1 = 36'hzzzzzzzzz;

//Disable audio codec.

//assign AUD_DACDAT = 1'b0;

//assign AUD_XCK    = 1'b0;

//Disable DRAM.

assign DRAM_ADDR  = 12'h0;

assign DRAM_BA_0  = 1'b0;

assign DRAM_BA_1  = 1'b0;

assign DRAM_CAS_N = 1'b1;

assign DRAM_CKE   = 1'b0;

assign DRAM_CLK   = 1'b0;

assign DRAM_CS_N  = 1'b1;

assign DRAM_DQ    = 16'hzzzz;

assign DRAM_LDQM  = 1'b0;

assign DRAM_RAS_N = 1'b1;

assign DRAM_UDQM  = 1'b0;

assign DRAM_WE_N  = 1'b1;

//Disable Ethernet.

assign ENET_CLK   = 1'b0;

assign ENET_CS_N  = 1'b1;

assign ENET_CMD   = 1'b0;

assign ENET_DATA  = 16'hzzzz;

assign ENET_RD_N  = 1'b1;

assign ENET_RST_N = 1'b1;

assign ENET_WR_N  = 1'b1;

//Disable flash.

assign FL_ADDR  = 22'h0;

assign FL_CE_N  = 1'b1;

assign FL_DQ    = 8'hzz;

assign FL_OE_N  = 1'b1;

assign FL_RST_N = 1'b1;

assign FL_WE_N  = 1'b1;

//Disable LCD.

assign LCD_BLON = 1'b0;

assign LCD_DATA = 8'hzz;

assign LCD_EN   = 1'b0;

assign LCD_ON   = 1'b0;

assign LCD_RS   = 1'b0;

assign LCD_RW   = 1'b0;

//Disable OTG.

assign OTG_ADDR    = 2'h0;

assign OTG_CS_N    = 1'b1;

assign OTG_DACK0_N = 1'b1;

assign OTG_DACK1_N = 1'b1;

assign OTG_FSPEED  = 1'b1;

assign OTG_DATA    = 16'hzzzz;

assign OTG_LSPEED  = 1'b1;

assign OTG_RD_N    = 1'b1;

assign OTG_RST_N   = 1'b1;

assign OTG_WR_N    = 1'b1;

//Disable SDRAM.

assign SD_DAT = 1'bz;

assign SD_CLK = 1'b0;

//Disable SRAM.

assign SRAM_ADDR = 18'h0;

assign SRAM_CE_N = 1'b1;

assign SRAM_DQ   = 16'hzzzz;

assign SRAM_LB_N = 1'b1;

assign SRAM_OE_N = 1'b1;

assign SRAM_UB_N = 1'b1;

assign SRAM_WE_N = 1'b1;

//Disable VGA.

/*

assign VGA_CLK   = 1'b0;

assign VGA_BLANK = 1'b0;

assign VGA_SYNC  = 1'b0;

assign VGA_HS    = 1'b0;

assign VGA_VS    = 1'b0;

assign VGA_R     = 10'h0;

assign VGA_G     = 10'h0;

assign VGA_B     = 10'h0;

*/

//Disable all other peripherals.

//assign I2C_SCLK = 1'b0;

assign IRDA_TXD = 1'b0;

//assign TD_RESET = 1'b0;

assign TDO = 1'b0;

assign UART_TXD = 1'b0;

wire    VGA_CTRL_CLK;

wire    AUD_CTRL_CLK;

wire    DLY_RST;

assign  TD_RESET    =   1'b1;   //  Allow 27 MHz

assign  AUD_ADCLRCK =   AUD_DACLRCK;

assign  AUD_XCK     =   AUD_CTRL_CLK;

Reset_Delay         r0  (   .iCLK(CLOCK_50),.oRESET(DLY_RST)    );

VGA_Audio_PLL       p1  (   .areset(~DLY_RST),.inclk0(CLOCK_27),.c0(VGA_CTRL_CLK),.c1(AUD_CTRL_CLK),.c2(VGA_CLK)    );

VGA_Controller      u1  (   //  Host Side

.iCursor_RGB_EN(4'b0111),

.oAddress(mVGA_ADDR),

.oCoord_X(Coord_X),

.oCoord_Y(Coord_Y),

.iRed(mVGA_R),

.iGreen(mVGA_G),

.iBlue(mVGA_B),

//  VGA Side

.oVGA_R(VGA_R),

.oVGA_G(VGA_G),

.oVGA_B(VGA_B),

.oVGA_H_SYNC(VGA_HS),

.oVGA_V_SYNC(VGA_VS),

.oVGA_SYNC(VGA_SYNC),

.oVGA_BLANK(VGA_BLANK),

//  Control Signal

.iCLK(VGA_CTRL_CLK),

.iRST_N(DLY_RST)    );

wire [9:0]  mVGA_R;             //memory output to VGA

wire [9:0]  mVGA_G;

wire [9:0]  mVGA_B;

wire [18:0] mVGA_ADDR;          //video memory address

wire [9:0]  Coord_X, Coord_Y;   //display coods

////////////////////////////////////

//DLA state machine variables

wire reset;

reg [3:0] state;    //state machine

reg [30:0] x_rand;  //shift registers for random number gen

reg [28:0] y_rand;

wire seed_low_bit, x_low_bit, y_low_bit; //rand low bits for SR

reg [9:0] x_walker; //particle coords of random walker

reg [8:0] y_walker;

reg [7:0] rule;

reg [3:0] sum; //neighbor sum

////////////////////////////////////

/*From megaWizard:

module vga_buffer (

address_a, // use a for state machine

address_b, // use b for VGA refresh

clock_a,

clock_b,

data_a,

data_b,

wren_a,

wren_b,

q_a,

q_b);*/

// Show m4k on the VGA

// -- use m4k a for state machine

// -- use m4k b for VGA refresh

wire mem_bit ; //current data from m4k to VGA

reg disp_bit ; // registered data from m4k to VGA

wire state_bit ; // current data from m4k to state machine

reg we ; // write enable for a

reg [18:0] addr_reg ; // for a

reg data_reg ; // for a

vga_buffer display(

.address_a (addr_reg) ,

.address_b ({Coord_X[9:0],Coord_Y[8:0]}), // vga current address

.clock_a (VGA_CTRL_CLK),

.clock_b (VGA_CTRL_CLK),

.data_a (data_reg),

.data_b (1'b0), // never write on port b

.wren_a (we),

.wren_b (1'b0), // never write on port b

.q_a (state_bit),

.q_b (mem_bit) ); // data used to update VGA

// make the color white

//reg [9:0] color_r = {8{disp_bit}} ;

//reg [9:0] color_g = {10{disp_bit}} ;

//reg [9:0] color_b = {2{disp_bit}} ;

assign  mVGA_R = {2{disp_bit}};

assign  mVGA_G = {10{disp_bit}};

assign  mVGA_B = {2{disp_bit}};

// DLA state machine

assign reset = ~KEY[0];

HexDigit H0(HEX0, rule[0]);

HexDigit H1(HEX1, rule[1]);

HexDigit H2(HEX2, rule[2]);

HexDigit H3(HEX3, rule[3]);

HexDigit H4(HEX4, rule[4]);

HexDigit H5(HEX5, rule[5]);

HexDigit H6(HEX6, rule[6]);

HexDigit H7(HEX7, rule[7]);

assign LEDR[7:0] = SW[7:0];

assign LEDG[0] = ~KEY[0];

assign LEDG[1] = ~KEY[0];

assign LEDG[2] = ~KEY[1];

assign LEDG[3] = ~KEY[1];

assign LEDG[4] = ~KEY[2];

assign LEDG[5] = ~KEY[2];

assign LEDG[6] = ~KEY[3];

assign LEDG[7] = ~KEY[3];

reg [7:0] current_state = 8'b11111111;

assign LEDR[17:10] = current_state[7:0];

reg reset_val;

assign LEDG[8] = reset_val;

reg [641:0] row_data = 642'd0; //Extra 2 are for zero-padding

reg [641:0] prow_data = 642'd0; //Extra 2 are for zero-padding

reg [8:0] current_row = 9'd0;

reg [9:0] current_col = 10'd1; //Start row pointer at the 1, due to zero padding.

wire [641:0] result;

reg print = 1'b1;

genvar index;

generate

for (index=1; index < 641; index=index+1)

begin: gen_code_label

Automaton A_inst(result[index], prow_data[index+1:index-1], rule);

end

endgenerate

//Randomly generated Seed for LFSRs

reg [639:0] prev_random_row = 640'hf9232a25a8c301011f64ff197a07037e15f2b9498b9dec83273c0dae73b1adbf1fa46164a15fa1057184b67f0c22d55ae3bf4f0c5a213cb2da6226b3520b3de83dea4fc9080c12544c4e2afb78206410;

wire [639:0] random_row;

generate

for (index=0; index < 20; index=index+1)

begin: gen_code_label2

Random32 R_inst(random_row[(32*index + 31):(32*index)], prev_random_row[(32*index + 31):(32*index)]);

end

endgenerate

//Automaton Tester

//state names

parameter init=4'd0, test1=4'd1, test2=4'd2, test3=4'd3, test4=4'd4, test5=4'd5, test6=4'd6,

new_row=4'd7, new_row2=4'd8, chill=4'd9,

chill2=4'd10, init2=4'd11, draw_walker1=4'd12, draw_walker2=4'd13 ;

always @ (negedge VGA_CTRL_CLK)

begin

// register the m4k output for better timing on VGA

// negedge seems to work better than posedge

disp_bit <= mem_bit;

end

always @ (posedge VGA_CTRL_CLK) //VGA_CTRL_CLK

begin

// register the m4k output for better timing on VGA

//disp_bit <= mem_bit;

if (reset)      //synch reset assumes KEY0 is held down 1/60 second

begin

reset_val <= 1'b1;

//clear the screen

addr_reg <= {Coord_X[9:0],Coord_Y[8:0]} ;   // [17:0]

we <= 1'b1;                             //write some memory

data_reg <= 1'b0;                       //write all zeros (black)

//read SW0:7

rule <= SW[7:0];

row_data <= 642'd0;

prow_data <= 642'd0;

if (SW[17]) begin

row_data <= {1'b0, prev_random_row, 1'b0}; //The random rows are 640 bits and each row needs to be 642

end

else begin

row_data[320] <= 1'b1;

end

state <= init;  //first state in regular state machine

end

//begin state machine to modify display

else

begin

reset_val <= 1'b0;

case(state)

// next three states write the inital dot

init: //write a single dot in the middle of the screen

begin

we <= 1'b1 ;

addr_reg <= {10'd320,9'd0} ;    //(x,y)

//write a white dot in the middle of the screen

data_reg <= 1'b1 ;

print = 1'b1;

current_col <= 10'd1;

current_row <= 9'd0;

current_state <= 8'b10000000;

state <= test1 ;

end

//

test1:

begin

we <= 1'b0;

//addr_reg <= {10'd250,9'd0};

//data_reg <= compute;

if (print == 1'b1) begin

addr_reg <= {current_col,current_row};

data_reg <= row_data[642-current_col];

//color_r <= {prow_data[current_col+1:current_col-1]{disp_bit}};

//color_g <= {prow_data[current_col+1:current_col-1]{disp_bit}};

//color_b <= {prow_data[current_col+1:current_col-1]{disp_bit}};

end

current_state <= 8'b01000000;

state <= test2;

end

//wait for write

test2:

begin

we <= 1'b1;

current_state <= 8'b00100000;

state <= test3;

end

//updating pixels

test3:

begin

we <= 1'b0;

if (current_col >= 10'd640) begin

current_col <= 10'd1;

if (current_row >= 9'd480) begin

//current_row <= 9'd0;

print <= 1'b0;

state <= chill;

end

else begin

//if ( ~KEY[2] )

//begin

current_row <= current_row + 9'd1;

state <= new_row;

//end

//else begin

//state <= test1;

//end

end

end

else begin

current_col <= current_col + 10'd1;

state <= test1;

end

current_state <= 8'b00010000;

//addr_reg <= {10'd250,9'd100};

//data_reg <= compute;

end

//starting a new row of pixels

new_row:

begin

if (print == 1'b1) begin

prow_data <= row_data;

end

current_state <= 8'b00001000;

state <= new_row2;

end

new_row2:

begin

if (print == 1'b1) begin

row_data <= result;

end

current_state <= 8'b00000100;

state <= test1;

end

chill:

begin

//just chill

if (~KEY[3])

begin

state <= chill2;

end

prev_random_row <= random_row;

current_state <= 8'b00000010;

//if (state != test1) begin

//state <= chill;

//end

end

chill2:

begin

if (KEY[3])

begin

print <= 1'b1;

current_row <= 9'd0;

current_col <= 10'd1;

state <= test1;

end

else begin

state <= chill2;

end

current_state <= 8'b00000001;

end

default:

begin

state <= chill;

end

endcase

end // else if ( KEY[3])

end // always @ (posedge VGA_CTRL_CLK)

endmodule //top module

//////////////////////////////////////////////

// Larger Automaton

module Automaton(outpixel, inpixels, rule);

input [2:0] inpixels;

output outpixel;

reg outpixel;

input [7:0] rule;

always @ (inpixels, rule)

begin

case (inpixels)

3'd0: outpixel <= rule[0];

3'd1: outpixel <= rule[1];

3'd2: outpixel <= rule[2];

3'd3: outpixel <= rule[3];

3'd4: outpixel <= rule[4];

3'd5: outpixel <= rule[5];

3'd6: outpixel <= rule[6];

3'd7: outpixel <= rule[7];

default: outpixel <= 1'b0;

endcase

end

endmodule

//////////////////////////////////////////////

// Random Number Generator

module Random32(out, in);

input [31:0] in;

output [31:0] out;

reg [31:0] out;

reg newbit;

always @ (in)

begin

newbit = in[31] ^ in[21] ^ in[1] ^ in[0];

out = {in[30:0], newbit};

end

endmodule

//////////////////////////////////////////////

// Decode one hex digit for LED 7-seg display

module HexDigit(segs, num);

input [3:0] num ;       //the hex digit to be displayed

output [6:0] segs ;     //actual LED segments

reg [6:0] segs ;

always @ (num)

begin

case (num)

4'h0: segs = 7'b1000000;

4'h1: segs = 7'b1111001;

4'h2: segs = 7'b0100100;

4'h3: segs = 7'b0110000;

4'h4: segs = 7'b0011001;

4'h5: segs = 7'b0010010;

4'h6: segs = 7'b0000010;

4'h7: segs = 7'b1111000;

4'h8: segs = 7'b0000000;

4'h9: segs = 7'b0010000;

4'ha: segs = 7'b0001000;

4'hb: segs = 7'b0000011;

4'hc: segs = 7'b1000110;

4'hd: segs = 7'b0100001;

4'he: segs = 7'b0000110;

4'hf: segs = 7'b0001110;

default segs = 7'b1111111;

endcase

end

endmodule

///////////////////////////////////////////////

////////// end of file //////////////////////////

c "Efficient Shift Registers, LFSR Counters, and Long Pseudo-Random Sequence Generators", Xilinx, XAPP 052 July 7,1996, http://www.xilinx.com/support/documentation/application_notes/xapp052.pdf